Thomas Heffron

Test #1 – Extra Points

Due Date: 18 Jun 2002

CS481, Inst.: Dr. Stach

- Q: Why do the ADD and SUB both have opcodes of 000000?

A: It makes sense that ADD and SUBTRACT opcodes are 000000 because SUBTACT operation is simply the ADDition of the 2s-compliment of the subtrahend (or third MIPS operand). So, it makes sense to send the same opcode with a different ‘function’ to the processor.

- Q: Why are all the R-type opcodes 000000?

A: Because the arithmetic instructions (on registers) are the most common case, we like to make them fast. All register-type instructions can have an opcode of 000000 because the processor can read this and know that the next 15 bits are read as 3 registers (2 operation and 1 destination) and that the last 6 bits are the function that defines the arithmetic operation on the registers. If the opcode is not 000000 the processor knows that it should read the next 26 bits differently.